Quantum Checks

Copyright (C) By Diego A. Padula

All rights reserved.

Documentation version 1.00B

Date of update: 08/11/1997

________________________________________________________________________

Table of contents

I. Purpose of the Quantum Checks

1. Parts of the package

2. Portability of the checks

II. License

III. About The Free Hardware Guide

IV. The Checks

1. RAMchk: On checking RAM

2. Drvchk: On checking storage devices

3. CPUchk: On checking the microprocessor

4. CMOSchk: On checking the BIOS' CMOS

5. Crashchk: On checking system reliability

1. Comparative chart

V. Contacting The Author

________________________________________________________________________

I. Purpose of the Quantum Checks

Today computer market is very wide and competitive, and as a result

of this, hardware of various brands and types is available, making

sometimes the task of selection harder to the purchaser. The

intention, then, is to provide you with a set of performance checks

ready to run and a set of comprehensive documentation, about almost

all the usually-buyed PC hardware pieces.

Although the documentation is addressed to the "medium level" user,

technically advanced users and professionals would find the checks

included in the package of practical use.

1. Parts of the package

This package is divided in two parts. The Checks, a set of

executable binary programs that perform various verifications and

then give back the results indicating certain performance or certain

information regarding some hardware aspect. These checks are mainly

intended to test the operating performance of a certain device (a

CD-ROM unit for example) with the hope that you would be able

to determine if the rates given are acceptable for your standards.

Thus, it can be said that some of the checks included perform as

benchmarks while others just give up some information.

Documentation is also provided, in which some issues related to

hardware, software performance, reliability, and compatibility can

be found. This, sometimes can be useful to decide which equipment to

buy. A few short technical briefings can be found in the

documentation.

2. Portability of the checks

Most of the checks included in this package are written for

portability, although preserving adequate quality standards. Anyway

the checks are mainly available in DOS EXE binary executable

form. Some of the checks have been successfully ported to other

systems than DOS although these are not included on the standard

package, if you're interested on them feel free to contact the

author.

________________________________________________________________________

II. License

The Quantum Checks is a package maintained By Diego A. Padula.

All the material included is copyrighted (C)1997 by Diego A. Padula

unless expressly stated.

The Free Hardware Guide is Copyright (C)1997 By Diego A. Padula.

The Quantum Checks are distributed as FREE software (not public

domain). Anyone is free to copy/distribute/use the package provided

that:

* It remains unmodified and complete as the original version

(included but not limited to: this documentation and the checks).

Derivative works or partial distributions are not permitted.

* No fee is requested to the end user. Only modest (discrete) fees

for costs of production and/or distribution are allowed (for

example: CD compilations).

* You will NOT reverse engineer, disassemble, or modify any of the

binary executables distributed within this package.

* You accept that The Author of the Quantum Checks is not

responsible for the use or misuse of the programs included in the

package. The checks have been designed with the hope that they will

be useful, although the author will not be responsible for any

damage that they may cause. No warranty is given (express or

implied).

Future versions of the Quantum Checks, could well be under a

different License.

If you don't understand any part of this License, please contact

The Author before using, distributing or copying this package.

________________________________________________________________________

III. About The Free Hardware Guide

The Free Hardware Guide has mainly two purposes: to bring a general

description of the currently used technologies within the PC

industry, and to aid the buyer in the task of acquiring the

appropriate hardware for the required standards. This guide can be

acquired separately or with the Quantum Checks, and is provided for

a better understanding of how the checks work.

________________________________________________________________________

IV. The Checks

Five checks are provided within this release of the Quantum Checks.

Most of these checks are weak in the aspect of certainly measuring

a specific device performance (for example, the RAMchk measures

general memory access performance, including caching), but can be

anyway a good start point for basic benchmarking (note that the

objective of the checks is not to benchmark specifically, but to

provide general tools for detecting and analyzing overall system

performance).

1. RAMchk: On checking RAM

This check is intended to measure read/write RAM performance. What

it basically does is: book some memory, read and write (copy) it

various times, and measure the time the copying takes.

Note that currently if it's true that RAMchk works writing and

reading to the main storage DRAM memory, the cache memory may affect

the measured results. That's why currently RAMchk does not check the

main storage DRAM precisely, but the whole I/O performance.

2. Drvchk: On checking storage devices

Usually Drvchk is useful to measure mass-storage devices performance

like CD-ROMs and has been mainly designed with this purpose. Any

drive that can be high-level accessed may be used with Drvchk; this

is because Drvchk does not access the device in a device-specific

(low-level) way. This has not only the advantage that it will work

with any drive, but also the portability benefits. The only drawback

is that system performance (like CD-ROM drivers, OS structures,

etc.) may strongly affect the performance measures.

The non-device specific policy means that you should be able to read

files from the disk/disc within the drive you specify. On most

systems if you want to measure the read-performance of a CD-ROM

alike drive, you will need data tracks for this purpose, since

usually non-data tracks (sound for example) are not accessed as

files by most OSes.

What Drvchk basically does is, once the parameters have been

specified, seeks for a file of about 500kb or more in the main or

first tree-level directories of the disk/disc, if found performs a

read performance check. The forced mode (-f option) is usually

useful when the media does not contain a 500+KB file; thus using

this option will only require a 153600 bytes file.

The program uses the syntax: Drvchk <drive> <-f> <-d>, where

drive: is the device on which the check should be performed.

-f: forced or fast check.

-d: show which archive is being used to perform the check.

3. CPUchk: On checking the microprocessor

CPUchk is currently not intended to measure processing performance.

Programs with such capabilities can be widely found. The goal of

CPUchk is to bring basic info about the CPU chip.

* Processor detection. In this field the program returns the

processor it has detected. The detection is performed on a try

basis, so what you get here is what you probably have. This program

detects up to 80686, further processors will not be detected by the

current version -- the latest IA-32 family processor is the Pentium

II, which is usually considered a 80686, Intel has announced that

the next processor they will release will be the Merced, an IA-64

chip.

* CPU Identification. Here the program gives the CPUID field. The

CPUID stuff was first implemented in the Intel Pentium processor.

This is a 12 bytes character array, which after execution of the

CPUID code (0Fh, A2h), the text-id is returned in EBX, EDX and ECX

(four bytes in each one, to form a 12 bytes field). If no CPUID

support is found the program returns "not available."

* Coprocessor detection. The program tries to find what co-processor

is installed (if any), the possible values are: None, 8087, 80287,

80387, and Build-in (reported when a 80486+ processor is found).

* Feature bits. The feature bits describe the CPU capabilities. Same

CPUs have equal feature bits. This field is usually useful to detect

MMX support on a Pentium, since the feature bits are not the same

than in a non-MMX Pentium.

4. CMOSchk: On checking the BIOS' CMOS

The current version of CMOSchk shows the basic information that is

stored in the CMOS. The sanity-check diagnostic is not performed by

the program, it relies on a byte status found in the CMOS.

The CMOS data is structured as an array of bytes in a static way.

Access to this information is gain through the usage of I/O ports

70h and 71h. When an offset position is output to the port 70h, the

corresponding byte in that position is placed in the port 71h.

5. Crashchk: On checking system reliability

This simple program is intended to "measure" the reliability of your

Operating System (OS). Versions of almost all the tests are

available for other systems like Linux and OS/2, if you are

interested on them contact me.

* The Overflow resistance test. This test produces a basic overflow.

This is, try to move/copy a word variable into a byte variable. Most

systems can resist this. This is one of the most common problems

that cause system crashes.

* The Video mad crash. This may become a funny thing. It tries to

quickly switch to a set of video modes using the BIOS (int 10h). On

some graphical interfaces this may cause discontroll, and could be

hard to restore the graphical terminal to it's correct mode. A

system must be prepared to handle these kind of illegal graphic-mode

switches.

* Sudden bootstrap. It tries to perform a bootstrap by using int

19h, in modern systems such a sudden bootstrap could be very risky,

since cache pools are not flushed, filesystems are not properly

unmounted, and the system is not shutdown. Almost every system

resists this.

* System memory trashing attempt. This test pretends to demonstrate

how many systems that say to run in full protected mode, and thus

guarantee system crash immunity are usually fakes. It basically

tries to copy some part of the memory in the current "segment" to

the start of the memory. This usually causes a crash on weak

systems, and similar causes are very common in everyday crashes.

5.1 Comparative chart

All these tests have been conducted with the 16-bit DOS version of

Crashchk, in a DOS compatible environment.

Test \ OS          |  Classic DOS  |  OS/2 Warp VDM  |  Windows95

-------------------+---------------+-----------------+-------------

1-Overflow resist. |     CRASH     |        -        |      -

2-Video mad crash  |       -       |        -        |    FUNNY

3-Sudden bootstrap |   BOOTSTRAP   |        -        |      -

4-S. memory trash  |     CRASH     |        -        |    CRASH

As you can see the Crashchk running on a VDM (Virtual DOS Machine)

under OS/2 Warp (Version 4), has shown full immunity to all kind of

the crashes produced by the current version of Crashchk. The Classic

DOS (6.0 version) has got crashed and bootstrapped, and the version

of Crashchk running on a DOS window under Windows95 completely

crashed the system when tried to trash the system memory (this shows

the weakness and low reliability of windows systems), and produced

funny effects while the video mad crash test was running (the

program was able to change the whole GUI resolution). The "-" means

that everything behaved correctly (no system crash was produced).

________________________________________________________________________

V. Contacting The Author

If you are interested in contacting me, please feel free to do it.

I will accept your suggestions, comments, corrections or any

information you can supply. If you want an updated version of this

package you can also request it directly from me.

E-mail: DAP@siscor.bibnal.edu.ar (please note the up-case)

Diego A. Padula

Formosa 157, 3-12

CP1424, Cap. Fed.

Buenos Aires

Argentina